## Lab Objectives

By the end of this lab, students will be able to:

- Install the XIAO nRF52840 Sense board package.

- Install the IMU library.

- select the correct board and serial port.

- Send text from the XIAO to the computer.

- Read accelerometer and gyroscope measurements.

- Display IMU measurements on the Arduino Serial Monitor.

# Part 1: Install the Required Board Package and Library

## Step 1: Add the Seeed Board Manager URL

- Open Arduino IDE.

- Select:

File → Preferences

- Find Additional Boards Manager URLs.

- Add the following address:

https://files.seeedstudio.com/arduino/package_seeeduino_boards_index.json

- Click OK.

## Step 2: Install the XIAO Board Package

- Select:

Tools → Board → Boards Manager

- Search for:

Seeed nRF52

- Install: both NRF boards for this lab, use the mbed-enabled board package.

## Step 3: Select the Board

Connect the XIAO nRF52840 Sense to the computer using the USB cable.

In Arduino IDE, select:

Tools → Board → Seeed nRF52 mbed-enabled Boards

Then select:

Seeed XIAO nRF52840 Sense Plus

Be careful not to select the normal XIAO nRF52840 board. The normal board does not contain the onboard IMU.

## Step 4: Select the Serial Port

Select:

Tools → Port

Choose the port connected to the XIAO. On Windows, it will normally appear as something similar to:

COM3
COM4
COM5

The port number may be different on each computer.

### Checkpoint 1

Before continuing, confirm that:

- The board package is installed.

- Seeed XIAO nRF52840 Sense Plus is selected.

- A serial port is visible and selected.

## Step 5: Install the IMU Library

Open:

Sketch → Include Library → Manage Libraries

Search for:

Seeed Arduino LSM6DS3

Install the library published by Seeed Studio.

### Alternative ZIP Installation

When the library does not appear in Library Manager:

- Download the Seeed_Arduino_LSM6DS3 ZIP file.

- In Arduino IDE, select:

Sketch → Include Library → Add .ZIP Library

- Select the downloaded ZIP file.

# Part 2: Test Serial Communication

Before using the IMU, confirm that the XIAO can send information to the computer.

## Step 1: Create a New Sketch

Select:

File → New Sketch

Delete the existing code and enter:

void setup() {
  // Start USB serial communication
  Serial.begin(115200);

  // Wait until the Serial Monitor is opened
  while (!Serial) {
    delay(10);
  }

  Serial.println("XIAO nRF52840 Sense is connected!");
}

void loop() {
  Serial.println("Hello from the XIAO");
  delay(1000);
}

## Step 2: Open the Serial Monitor

Open:

Tools → Serial Monitor

Alternatively, click the Serial Monitor icon in the upper-right corner of Arduino IDE.

Set the baud rate to:

115200

You should see:

XIAO nRF52840 Sense is connected!
Hello from the XIAO
Hello from the XIAO
Hello from the XIAO

A new message should appear approximately once every second.

### Checkpoint 2

Serial communication is working when:

- The sketch uploads successfully.

- The correct COM port is selected.

- Messages appear in the Serial Monitor.

- The baud rate is set to 115200.

# Part 3: Start the IMU

The onboard IMU measures:

| Measurement | Axes | Typical unit |
| --- | --- | --- |
| Acceleration | X, Y and Z | g |
| Angular velocity | X, Y and Z | degrees per second |

The accelerometer detects movement and gravity. The gyroscope detects rotational movement.

Down load Adafruit library And add it: https://github.com/adafruit/Adafruit_LSM6DS

## Step 3.1: Upload the IMU Program

Create a new sketch and enter the following code:

## Code attached

## Step 3.2: Observe the Measurements

- Upload the program.

- Open the Serial Monitor.

- Set the baud rate to 115200.

- Keep the board stationary.

- Move the board in different directions.

- Rotate the board around each axis.

Example output:

Starting XIAO nRF52840 Sense IMU...
IMU started successfully.

Accelerometer
AX: 0.021 g    AY: -0.014 g    AZ: 1.002 g
Gyroscope
GX: 0.31 deg/s    GY: -0.28 deg/s    GZ: 0.15 deg/s
------------------------------

When the board is resting flat, one accelerometer axis will normally measure approximately 1 g because of gravity.

When the board is rotated, the gyroscope values should change.

### Checkpoint 3

The IMU is operating correctly when:

- IMU started successfully appears.

- Six values appear on the Serial Monitor.

- Accelerometer values change when the board is moved.

- Gyroscope values change when the board is rotated.

# Troubleshooting

## The Port Does Not Appear

- Disconnect and reconnect the USB cable.

- Try another USB cable.

- Try another USB port.

- Press the reset button once.

- If necessary, press the reset button twice quickly to enter bootloader mode.

- Check Tools → Port again.

## The Program Does Not Upload

Confirm that the selected board is:

Seeed XIAO nRF52840 Sense Plus

under:

Seeed nRF52 mbed-enabled Boards

Also close the Serial Monitor before uploading again.

## Nothing Appears in the Serial Monitor

Check the following:

- Correct COM port selected

- Baud rate set to 115200

- Serial Monitor opened after uploading

- USB cable supports data

- Board reset once after uploading

# Student Exercise

Modify the IMU program so that it:

- Reads the accelerometer every 100 ms.

- Displays only AX, AY and AZ.