# 1. Test the ECG signal

Estimated time: 30–40 minutes

## A1. Connect the AD8232

| AD8232 pin | XIAO nRF52840 pin | Function |
| --- | --- | --- |
| 3.3V | 3V3 | Power |
| GND | GND | Common ground |
| OUTPUT | D0 / A0 | Analog ECG signal |
| LO+ | Not connected in this test | Optional lead-off detection |
| LO− | Not connected in this test | Optional lead-off detection |
| SDN | Not connected | Optional shutdown input |

## A2. Place the electrodes

RA: right upper chest or right forearm.

LA: left upper chest or left forearm.

RL: lower-right abdomen or right leg.

For the cleanest first test, use chest placement, fresh adhesive electrodes, and remain completely still.

## A3. Upload and view the signal

1.  Open ECG_Test.ino.

2.  Select the correct XIAO nRF52840 board and COM port.

3.  Upload the sketch.

4.  Open Tools → Serial Plotter and select 115200 baud.

5.  Observe both Raw and Filtered signals for at least 20 seconds.

# Part B — Train and export the ECG model

## B1. Open the Colab notebook

1.  Upload ECG_Training.ipynb to Google Drive.

2.  Open it with Google Colab.

3.  Select Runtime → Change runtime type → T4 GPU if available.

4.  Run the notebook cells in order.

## B2. Select the number of classes

NUMBER_OF_CLASSES = 4 # Recommended for this lab

The four default AAMI heartbeat groups are:

| Class | Meaning | Example source annotations |
| --- | --- | --- |
| N | Normal-type beats | N, L, R, e, j |
| S | Supraventricular ectopic beats | A, a, J, S |
| V | Ventricular ectopic beats | V, E |
| F | Fusion beats | F |
| Q (optional) | Paced or unclassifiable beats | /, f, Q |

## B3. Understand the preprocessing

The MIT-BIH recordings are read at 360 Hz.

Each annotated beat is cropped from 0.30 s before the R peak to 0.50 s after it.

The beat is resampled to 200 samples at 250 Hz to match the XIAO acquisition rate.

The mean is removed and the beat is divided by its largest absolute amplitude.

Training and testing use different patient records to reduce data leakage.

## B4. Train and evaluate

1.  Inspect the example waveform plotted for each class.

2.  Run the training cell and wait for early stopping.

3.  Record the floating-point test accuracy.

4.  Record precision, recall, and F1 score for N, S, V, and F.

5.  Inspect the confusion matrix. Pay particular attention to S and F, which have fewer examples.

6.  Convert the model to fully quantized int8 TensorFlow Lite and evaluate it again.

## B5. Download deployment files

model.h — the int8 TensorFlow Lite model stored as a C array.

ecg_model_config.h — sample rate, window size, class count, quantization parameters, and labels.

# Part C — Deploy the model and advertise results by BLE

Estimated time: 40–50 minutes

## C1. Prepare the XIAO project

ECG_XIAO_BLE_Beacon_Classifier/ ├── ECG_XIAO_BLE_Beacon_Classifier.ino ├── model.h └── ecg_model_config.h

1.  Replace the supplied model.h and ecg_model_config.h with the files generated by your Colab run.

2.  In Boards Manager, install Seeed nRF52 mbed-enabled Boards.

3.  Select the XIAO nRF52840 Sense/Sense Plus board from the mbed-enabled section.

4.  Install Chirale_TensorFlowLite and ArduinoBLE from Library Manager.

5.  Keep the AD8232 OUTPUT connected to D0/A0.

6.  Upload ECG_XIAO_BLE_Beacon_Classifier.ino and open Serial Monitor at 115200 baud.

## C2. Observe startup and classification

TensorFlow Lite Micro initialized. Model bytes: ... Input scale: ... Input zero point: ... Output classes: 4 BLE beacon: CALIBRATING ¦ seq=1 R-peak detector calibrated. Initial threshold: ... BLE beacon: NOT DETECTED ¦ seq=2

When a valid beat is detected and classified, the monitor prints all probabilities:

Beat @ 8.24 s ¦ N:82.4% ¦ S:5.1% ¦ V:10.8% ¦ F:1.7% => N Normal-type (82.4%) BLE beacon: DETECTED ¦ class=N ¦ confidence=82% ¦ bpm=74 ¦ seq=3

## C3. BLE status logic

| Status | Meaning | When it is advertised |
| --- | --- | --- |
| CALIBRATING | R-peak detector is learning the initial signal/noise levels. | Immediately after startup. |
| NOT DETECTED | No valid classification is available. | After calibration or after five seconds without a valid classification. |
| DETECTED | A beat was accepted and classified. | After successful TinyML inference. |

## C4. BLE beacon packet

| Byte(s) | Field | Values |
| --- | --- | --- |
| 0–1 | Company ID | 0xFFFF, laboratory use |
| 2–4 | Magic identifier | ASCII E C G |
| 5 | Protocol version | 1 |
| 6 | Status | 0=Not detected, 1=Detected, 2=Calibrating |
| 7 | Class | 0=N, 1=S, 2=V, 3=F, 4=Q, 255=None |
| 8 | Confidence | 0–100% |
| 9 | Heart rate | BPM; 0 means unavailable |
| 10–11 | Sequence number | Little-endian counter |

# Part D — Receive the ECG beacon on the XIAO ESP32-S3

## D1. Configure the ESP32-S3

Board: Seeed Studio XIAO ESP32S3.

USB CDC On Boot: Enabled.

USB Mode: Hardware CDC and JTAG.

Serial Monitor: 115200 baud.

Use the BLE library included with the ESP32 board package; NimBLE is not required.

## D2. Upload the receiver

1.  Open ESP32S3_ECG_Beacon_Receiver/ESP32S3_ECG_Beacon_Receiver.ino.

2.  Select the XIAO ESP32-S3 board and port.

3.  Upload the sketch.

4.  After upload, re-check the COM port because native USB may reconnect using a new port.

5.  Open Serial Monitor at 115200 baud and press Reset once if necessary.

6.  Power the XIAO nRF52840 transmitter and keep it near the receiver.

## D3. Expected receiver output

Scanning for ECG-XIAO... Total BLE devices seen: 8 ======================================== Module: ECG-XIAO Address: XX:XX:XX:XX:XX:XX RSSI: -47 dBm Packet number: 3 Status: DETECTED Class code: N Classification: Normal-type Confidence: 82% Estimated heart rate: 74 BPM Raw manufacturer data: FF FF 45 43 47 01 01 00 52 4A 03 00 ========================================